EMERGENCY RESPONSE PROCEDURES FOR BUS OR COACH FIRES

S	A	F	E

STOP	ALERT	FINALISE	EVACUATE

If the fire alarm or pressure monitor activates, or you see, smell or are advised of smoke or flames, you must adhere to the following procedures to ensure the safety of both yourself and any passengers in or around the vehicle.

Step 1	Remain Calm.

Step 2	Pull vehicle over immediately to the near side lane/kerb ensuring that it is safe and out of danger from other road users, trees and buildings.

Step 3	Secure vehicle by applying the park or handbrake.

Step 4	Open all doors and SHUT down engine as soon as practicable.

Step 5.	Instruct passengers to disembark in an orderly and safe manner and move away to the side to a safe location.

Step 6	Instruct passengers to disembark in an orderly and safe manner and move away to a safe location.

Step 7	If vehicle is fitted with master isolator switch, this must be switched off.

Step 8	If vehicle is fitted with a fuel isolator switch, this must be switched off.

Step 9.	Access vehicle fire extinguisher and attempt to contain fire (if the fire is contained to the engine bay or luggage compartment, you must NEVER open the engine hatch to attempt to extinguish fire).

Step 10. If access to exit doors is restricted due to fire, then instruct a responsible person to utilise emergency window hammers to break emergency windows to allow access from the side of the vehicle.

Step 11. Check the welfare of your passengers and assess any injuries.

Adapted from protocols provided by Brisbane City Council in the development of the Fire Mitigation Advisory, 2014, Bus Industry Confederation