Driver Pre-Departure and Operational Check Sheet

| Driver Pre-departure and Operational check sheet Pro-Forma |  |  |  |
| --- | --- | --- | --- |
| Items to be checked or observations required |  | Requires Rectification |  |
|  |  | Yes | No |
| A | Driver Pre-Departure Checks |  |  |
| 1 | Look for tyres that are ﬂat or partly ﬂat. |  |  |
| 2 | Check that tyres on dual wheels are not touching one another. |  |  |
| 3 | Make sure that no liquids are leaking onto the road, or into the engine compartment. |  |  |
| 4 | Water coming from the air conditioning system is okay – any other dripping, spray or misting is not. |  |  |
| 5 | Check that the exhaust brake is working correctly if your vehicle has one. |  |  |
| 6 | Check that the brakes release properly and that the air gauges are all in the normal position. |  |  |
| 7 | Check that there is no oil coming from the exhaust pipe and no debris in the pipe. |  |  |
| 8 | Check for excessive blue smoke from the exhaust – this could indicate the turbocharger is burning oil. |  |  |
| 9 | Check that the vehicle has an unexpired ﬁre extinguisher. |  |  |
| B | Checks When Operating the Bus |  |  |
| 1 | Check the temperature gauges for overheating. |  |  |
| 2 | Check the low tyre pressure monitor if your vehicle has one. |  |  |
| 3 | Check the air pressure gauge and/or warning light. Low air pressure can cause spring brakes to apply and bind. |  |  |
| 4 | Check the ABS warning light. |  |  |
| 5 | Check that the retarder is working properly. |  |  |
| 6 | Check for any alarms or alerts on the instrument panel, if in doubt seek advice on what the issue is. |  |  |
| C | End of Shift Checks |  |  |
| 1 | Record any problems in the vehicle’s defect card/sheet. |  |  |
| 2 | Tell a mechanic or the next driver if there are problems that require attention. |  |  |